## CMD IDE 064x Windows NT4 Driver version 3.0.8

CMD Technology Inc.

### 19 Morgan 	Voice:	(949) 454-0800

Irvine, CA 92618	FAX:	(949) 454-8314

Web Site:	http://www.cmd.com

#### Contents

##### Overview

- Applicable Hardware

- Installation

- New Features And Enhancements

- Corrections

- Known Restrictions

###### Overview

Driver File:		csa64xx.sys

Driver version:	3.0.8

Driver Dates:		6/07/2000

OS required to run:	Windows NT4.0

Other Files included:	CmdCfgNt.exe

###### Applicable Hardware

This release supports the following hardware:

CMD IDE 0648 for WHQL

- Installation

Follow the Microsoft driver update procedures.

- New Features and Enhancement

Enhancement:	Changed algorithm for setting the correct value for task file timing register.

- Corrections

Symptom:	Connect Richo CD-R/RW (model RW7060A) to the secondary channel master and any hard disk as secondary slave, after system boot up, driver can not see the hard disk.

Action:	This problem has been fixed.

Symptom:	Timing issue for 649 Rev. 2 – On some older motherboards, system hangs during boot.

Action:	Before sending any command to devices, set chip mode to PIO 0 (during boot time and after software reset).  Also change recovery count to 3 for PIO mode 0, 1, and 2 if controller chip is 649 Rev. 2.

- Known Restrictions

NT4 prohibits PIO and DMA on the same IDE channel.

CD-ROM and DVD-ROM running in MDMA2 mode is downgraded to MDMA1.

Connect Fujitsu hard drive (model MPD3043AT) and Toshiba CD-ROM on the same channel, OS can not detect hard drive or CD-ROM  (This might caused by devices’ load, driver can not fix this problem.  Avoid using this device combination.  If one wish to use this combination, the Fujitsu hard driver must be set to MDMA0 or PIO mode in order to function properly.)

When doing fresh NT4 installation from CD-ROM or diskette with Intel 840 motherboard system, the system crashes at the last stage if we choose to install network driver.  This also happened to Promise controller card and the onboard IDE controller.  There must be an issue with this motherboard which has an onboard network controller.

CD changer is not supported.

If the IDE controller is set to IDE mode and there is any device connected to the onboard non CMD IDE controller, during system time, NT4 will try to load both the default IDE driver and CMD driver.  This will cause a resource conflict and CMD driver will not be loaded.  To prevent this occur, set CMD IDE controller to RAID mode.